Office of Occupational Health and Safety, Animal Welfare and Environmental Protection

# Safety Guidelines

in accordance with Section 14 Gefahrstoffverordnung (directive on hazardous substances, GefStoffV)

Liquid Nitrogen (N2)

University of Würzburg

Last update:

Workplace/activity:

| ## Scope |  |
| --- | --- |
|  | Safe handling of liquid nitrogen (N2). Nitrogen is a colourless, odourless and tasteless gas that condenses to a colourless liquid at −196 °C. |
| ### Safety, health and environmental risks |  |
|  | - Asphyxiation: Nitrogen displaces oxygen from the ambient air → danger of asphyxiation, in particular when liquid nitrogen is being decanted or transferred from one container to another in a room that has poor ventilation. - Frostbite injuries or cold burns: When direct contact occurs. Eye contact may result in severe eye damage. |
| ### Safety practices |  |
|  | Personnel must receive training from a person in charge before being allowed to handle liquid nitrogen. - Decant or transfer liquid nitrogen only in suitable rooms with sufficient air changes. - Before entering liquid nitrogen rooms, always check the control panel of the ventilation system to ensure that the system is working properly. - Use only containers made specifically for cryogenic liquids. - Take precautions to prevent containers holding liquid nitrogen from tipping over; protect containers from exposure to shock or high temperatures. - Wear a face shield or safety glasses with side shields that provide sufficient protection; wear protective clothing (lab apron) and protective gloves. - When kept or handled in open containers, liquid nitrogen will cause oxygen to condense from the ambient air through an exchange of heat. This will lead to a gradual accumulation of strongly oxidising oxygen due to which spontaneous combustion may occur on contact with highly flammable materials. |
| What to do in hazardous situations |  |
|  | - When the oxygen monitoring system triggers an alert, leave the room as quickly as possible, allow plenty of fresh air into the room. - Always wait until the person in charge has given you the ‘all clear’ before returning to the affected room. |
| What to do after an accident, first aid |  |
|  | - When providing first aid, always ensure your own safety. - Immediately supply the victim with fresh air. - If the victim is not breathing, immediately administer artificial respiration (keep in mind to ensure your own safety). - Call emergency medical services. - Seek immediate medical attention for cold burns. Emergency number: 112 First aider: |

…………………………………….                                                   …………………………………………….

Date                                                                                          Signature

June 2021