# Safety Guidelines

Microbiological Safety Cabinets

Office of Occupational Health and Safety, Animal Welfare and Environmental Protection

University of Würzburg

Last update:

Workplace/activity:

| ## Scope |  |
| --- | --- |
|  | Safe use of class II microbiological safety cabinets (MSCs). |
| ### Safety, health, and environmental risks |  |
|  | Exposure to biological agents. |
| ### Safety practices |  |
|  | - MSCs do NOT provide protection from harmful gases or vapours. - Use key switch to turn on cabinet; cabinet must be on at least 15 minutes before work begins. Do not begin work until lights indicate safe work mode. - Avoid external draughts caused by opening doors, people entering room or walking past front of cabinet, etc. and keep windows and doors closed while work is ongoing. - Avoid disruption of laminar airflow: - avoid swift or vigorous arm movements, - avoid placing larger pieces of equipment in cabinet; if their use cannot be avoided, do not block any air vents or grills, - do not use Bunsen burners in cabinet; if an open flame is absolutely necessary for the work, use a burner that provides a flame only as long as a button is kept pressed. - Load cabinet with only the items you need for the procedure. Do not place more materials or equipment in the cabinet than needed for the procedure. - Always wear personal protective equipment: lab coat, disposable gloves, and goggles. - Avoid aerosol generation. - Always disinfect equipment and containers before removing them from cabinet. - After completion of work in MSC and before you turn off the unit, put cover lids on open cultures and place infectious items and materials in sealed containers. - Clean and disinfect cabinet surfaces after completion of work. When using flammable disinfectants (e.g. alcohols), always take appropriate measures to ensure that an explosive atmosphere is not allowed to form. |
| What to do in the event of malfunction |  |
|  | - When work is ongoing, always ensure green light indicates safe work mode and sash is at its proper position. - Always react to alarms (e.g. airflow or filter replacement alarms). - If the cabinet fails, immediately discontinue work. Immediately notify supervisor and biosafety officer. |
| What to do after an accident |  |
|  | - Seek help from first aiders. - Immediately call 112 to alert emergency responders. - Render first aid. - Immediately report accident. |
| Maintenance, waste disposal |  |
|  | Written approval must be obtained from head of lab, and the room disinfected, before any maintenance work is performed. |

……………………………………                                          ………………………………………….

Date						                                Signature

June 2021