Office of Occupational Health and Safety, Animal Welfare and Environmental Protection

# Safety Guidelines

Laboratory Ovens

University of Würzburg

Last update:

Workplace/activity:

| ## Scope |  |
| --- | --- |
|  | Safe drying of samples containing flammable solvents in laboratory ovens. |
| ### Safety, health, and environmental risks |  |
|  | - Solvent vapour build-up, exposure above permissible exposure limit. - Formation of explosive vapour-air mixtures. - Fire hazard if solvent is heated to a temperature near its flash point. - Heat burns from hot oven chamber, walls, and door. |
| ### Safety practices |  |
|  | - Always use vacuum or explosion-proof ovens. - Set temperature to below solvent’s flash point. - Inspect oven and, in particular, seals, hinges, and switches for signs of damage. |
| What to do in the event of malfunction |  |
|  | - If you notice anything abnormal, turn off oven and notify supervisor. - In the event of fire, attempt to extinguish with C02 extinguisher but only if you can do so without putting yourself in danger. - In the event of a release of significant amounts of solvent vapours, alert others in the affected area, open windows where appropriate, ventilate room. |
| What to do after an accident |  |
|  | - Move injured persons to a safe area and render first aid. - Call 112 to alert emergency responders. - Brief emergency responders. - Notify first aiders. |
| Maintenance, waste disposal |  |
|  | - All replacement parts must be original manufacturer or equivalent to original manufacturer parts. Always follow manufacturer’s maintenance schedule. |

…………………………………….                                                        ……………………………………………….

Date                                                                                           Signature

June 2021