# Safety Guidelines

Fume Hoods and Cupboards

Office of Occupational Health and Safety, Animal Welfare and Environmental Protection

University of Würzburg

Last update:

Workplace/activity:

| ## Scope |  |
| --- | --- |
|  | Safe use of fume hoods and cupboards (DIN 12924-1, DIN EN 14175). |
| ### Safety, health, and environmental risks |  |
|  | - Exposure to hazardous materials when front sash is open or if airflow is blocked with unnecessary materials or equipment, there is external air movement (draughts, turbulence in front of hood/cupboard), you are working with high heat loads in hood/cupboard. - Explosions caused by the release of significant amounts of combustible/flammable gases, vapours, particulates/dust, or aerosols. - Splash or projectile hazards when front sash is open. - Personal injury hazards if cable snaps and front sash drops. |
| ### Safety practices |  |
|  | - Before work begins and while work is ongoing, always ensure that fume hood/cupboard is functioning correctly, green light - exhaust airflow rate OK. - Always wear personal protective equipment (lab coat, goggles, and protective gloves). - Keep front sash closed; wherever reasonably practicable, position horizontal sliding sashes between operator and work in the hood/cupboard and slide sashes to minimise openings. - Load hood/cupboard with only the items you need for the procedure. - Do not place larger pieces of equipment in hood/cupboard. - Avoid rapid movements in front of the hood/cupboard. - Never lean into, or put your head inside, a fume hood/cupboard. - Never allow larger amounts of substance (gases, vapours) to be released inside hood/cupboard; wherever possible and reasonably practicable, capture and remove emissions at the point of release. - Never work with high heat loads in hood/cupboard (do not work with more than one Bunsen burner or more than one hot plate in a fume hood/cupboard). If you cannot avoid working with high heat loads, use a hood/cupboard specifically designed for high heat loads. |
| What to do in the event of malfunction |  |
|  | - If front sash is not moving smoothly or is otherwise broken, discontinue work and notify supervisor. - When an audio (horn) or visual (red light) alarm goes off, calmly cease potentially unsafe activities and stop using the hood/cupboard immediately. Explosion and hazardous material release hazards arising from low exhaust airflow. - If hood/cupboard is not functioning correctly, notify supervisor or Technical Maintenance Service Centre (phone: 31 84444). |
| What to do after an accident |  |
|  | - Seek help from first aider (name see ‘What to Do in the Event of an Accident’ poster). - Treat minor injuries immediately. - Call 112 to alert emergency responders. - Report accident to supervisor. |
| Maintenance |  |
|  | - Fume hoods and cupboards are subject to regular annual inspection and maintenance. All repairs and maintenance work must be performed by qualified personnel. |

…………………………………….                                                        ………………………………………………...

Date                                                                                                    Signature

June 2021