# Safety Guidelines

Compressed Gas Cylinders

Office of Occupational Health and Safety, Animal Welfare and Environmental Protection

University of Würzburg

Last update:

Workplace/activity:

| ## Scope |  |
| --- | --- |
|  | Safe storage and transportation of cylinders containing flammable, oxidising, toxic, corrosive, and/or inert compressed gases. |
| ### Safety, health, and environmental risks |  |
|  | - Highly toxic. - Fire, explosion, or toxic release hazards caused by leaking valves. - Gas cylinders may burst when heated in a fire; projectile hazards when gas is escaping rapidly. - Highly flammable. |
| ### Safety practices |  |
|  | - Always check a cylinder’s test expiry date before use; if that date has passed, do not use but return cylinder to supplier. - Appropriate hazard warnings must be clearly labelled on the cylinders (hazard and precautionary statements). - Cylinders must never be permitted to be exposed to high temperatures. - Cylinders must be stored in a vertical position and must be strapped or secured to prevent falling or rolling. - All cylinders must be stored with the protective caps in place. - When cylinders are being moved from one area to another, a suitable hand cart must be used, cylinders must be secured to the cart with a chain or belt, and the protective caps must be in place. - A gas mask with an appropriate filter must be kept available for immediate use at all times when transporting cylinders containing highly or extremely toxic gases. - Flammable gases (red cylinders) must be stored at least 2 metres from oxidisers (e.g. oxygen), e.g. at opposite ends of the storage area and separated by cylinders containing non-flammable gases. |
| What to do in the event of equipment failure or leakage |  |
|  | - Move injured persons to a safe area, but do not put yourself in danger. Alert everyone who might be in danger. - What to do in the event of a fire: Move gas cylinders to a safe area. If a cylinder cannot be moved to a safe area, go to a safe location and cool the cylinder with water until the fire brigade arrives. Alert emergency responders to the presence of compressed gas cylinders. - Nearest extinguisher: - Nearest gas mask: |
| What to do after an accident |  |
|  | - Eye contact: Rinse opened eye under running water for several minutes. After that, seek medical attention from an ophthalmologist. - Skin contact: Wash with plenty of water. Remove contaminated clothing. - Inhalation: Keep victim’s airway clear. Move victim into fresh air. Administer a dexamethasone spray (Auxiloson). When necessary: Perform mouth-to-mouth resuscitation. |
| Maintenance, waste disposal |  |
|  | Hazardous waste must be disposed of in properly labelled, appropriate containers. A request for disposal must be made at the Office of Occupational Health and Safety, Animal Welfare and Environmental Protection, Mr. Heiko Richter. |

…………………………………….                                                        ……………………………………………….

Date                                                                                                               Signature

June 2021