Office of Occupational Health and Safety, Animal Welfare and Environmental Protection

# Safety Guidelines

Ultraviolet Light

University of Würzburg

Last update:

Workplace/activity:

| ## Scope |  |
| --- | --- |
|  | Safe use of ultraviolet light. Ultraviolet (UV) light is invisible, high-energy light in the 1 to 400-nm wavelength region of the electromagnetic spectrum. UV light is also present in sunlight; most of it is absorbed in the atmosphere before it reaches the Earth’s surface. |
| ### Safety, health, and environmental risks |  |
|  | - Eye irritation and injury, blindness, skin irritation. - Respiratory tract irritation from high output UV lamps that cause oxygen to be converted into ozone. - Heat burns from hot UV lamps. |
| ### Safety practices |  |
|  | - Prevent unauthorised access to work area. - Always use UV lamps with the transformers that were delivered with them. - Avoid exposure of UV lamps to vibration or shock. - Avoid turning UV lamps on and off at frequent intervals. - UV lamps (high output lamps in particular) become very hot; always ensure they are cooled efficiently. - Always wear UV protective safety glasses when UV lamp is on! Always wear safety glasses designed for the specific wavelength and power of the UV emitting device being used. - Never look into a UV lamp while it is on! - Cover light sources with opaque (non-combustible) material, e.g. aluminium foil. - When working with high output ozone producing lamps, always work in a fume hood or use a suitable local extractor. |
| What to do in the event of malfunction |  |
|  | - Turn off device and disconnect it from power supply. - Immediately notify supervisor or other competent individual. - All malfunction diagnoses and repairs must be performed by qualified personnel. |
| What to do after an accident |  |
|  | - Call 112 to alert emergency responders. - If individuals suffered corneal flash burns, cover their eyes with a clean cloth and make sure they are taken to the University Eye Clinic for treatment. - Report accident. |
| Maintenance, waste disposal |  |
|  | UV lamps must be treated and disposed of as hazardous/special waste. The Technical Maintenance Service Centre is available to assist with disposal where needed. |

…………………………………….                                                        ……………………………………………….

Date                                                                                                     Signature

June 2021