Office of Occupational Health and Safety, Animal Welfare and Environmental Protection

# Safety Guidelines

Centrifuges

University of Würzburg

Last update:

Workplace/activity:

| ## Safety, health, and environmental risks |  |
| --- | --- |
|  | - Bursting of drum or rotor. - Fire or explosion. - Accidental flyaways. - Hazards arising from lack of machine stability. - Hazards arising from failure of automatic lid locking mechanism. |
| ### Safety practices |  |
|  | - Inspect centrifuge for visible signs of damage before use. - Always ensure that lid locking mechanism is functioning correctly and lid is properly secured. - Individuals operating centrifuges or performing maintenance work must be 18 or over (for exceptions see Section 17 VBG 7z regulations). - Never fill tubes above the maximum recommended by manufacturer, and never exceed maximum stated speed for any rotor. - Always ensure that rotor loads are evenly balanced. - Turn off centrifuge if machine is excessively out of balance (Section 19 VBG 7z regulations). - When operating centrifuges that are loaded with flammable or highly flammable samples and in which a potentially explosive atmosphere may form, always ensure that there is no fire or explosion hazard in the centrifuge (Section 21 VBG 7z regulations). - Do not open or remove housing cover for any purpose other than routine cleaning, and never open or remove cover during operation. |
| ### What to do in the event of malfunction |  |
|  | - If lid locking mechanism is not functioning correctly or machine is excessively out of balance, immediately discontinue operation and notify supervisor. - In the event of explosions or drum/rotor bursts, immediately notify Office of Occupational Health and Safety, Animal Welfare and Environmental Protection (phone: 31-82684). - All repairs and maintenance work must be performed by qualified contractors. |
| What to do after an accident |  |
|  | - Turn off centrifuge. - Move injured persons to a safe area. - Call 112 to alert emergency responders. - Render first aid. |
| Maintenance, waste disposal |  |
|  | #### Centrifuges must be inspected once a year in an assembled state and every three years in a disassembled state. All inspections must be carried out by qualified technicians. The inspection record must be kept with the machine. |

…………………………………….                                                        ……………………………………………….

Date                                                                                                    Signature

June 2021